CATIA V5 R18

3D Text Tool v3

Serge Isambert, Jacques Mazaud

April 2009

Contents

# Introduction……………………………………………3

- Administration & Settings	…………………………..3

  - Start-up drawing………………………………………………………..3

  - Environment variables…………………………………………………5

  - CATIA Settings…………………………………………………………7

  - Adding more fonts……………………………………………………...8

  - Using an icon to instantiate a Power Copy………….………………9

- Using the 3D Text tool………………………………11

  - 3D Text creation……………………………………………………....11

  - 3D Text modifications………………………………………………..14

# Introduction

# Designers sometimes need to enrich their industrial parts with engraved or embossed texts. Our  software Partner TYPE3 can achieve this and much more (industrial logo designs, artistic texts, serial numbers, legal mentions, etc,) but it’s too complex if the need is just to create texts in a Sketch.

# Today only a manual solution exists in CATIA V5. It consists in creating a text in a drawing, exporting it as a DXF file then re-importing this file, copying the lines, arcs and splines into a part Sketch and then using a Pad or a Pocket feature.

# The 3D Text tool detailed in this document automates this process. It is based on CATIA V5 standard features and Knowledgeware technology. We’ll first explain how to set and customize this tool and then we will see how to use it.

The 3D Text tool consists in one CATPart file containing a PowerCopy feature that creates 3D texts as solid extrusions or pockets. It is free and can be distributed and used free of charge.  To run the tool, a CATIA configuration containing Part Design 1 (PD1) license is needed.

- Administration & Settings

  - Start-up drawing

The 3D Text tool uses a drawing file to create temporary Texts (transparent step for users). Your CATIA administrator may have limited the number of fonts available thru the drafting standard file. However, for 3D Texts purpose, you might want to have more fonts available. Consequently, the 3D Text tool uses a start up drawing including a standard which may be different from your company one. Two cases have to be considered. Either you want to use for 3D Texts a font which is not in the allowed font list (see 2.1) or the needed fonts are all allowed by your CATIA administrator (see 2.2)

- Restricted fonts list

In this case it is necessary to create a new drafting standard that you will associate to the start up drawing file.

- Start CATIA V5 in administrator mode

- Use Tools/Standard command, drafting category and select one of the international standards (ISO, ANSI,etc) or your current company standard.

- Make your modifications in the Allowed Text Fonts chapter and click Save As New button to save your new standard. Save it as “3D Text.xml” for example (Figure 2.1).

- Create a new drawing (File/New command) and select the 3D Text standard (Figure 2.2).

- Let the drawing empty and save it in a specific directory.

Figure 2.1: creating a new drafting standard to allow more text fonts

Figure 2.2: creating a new drawing with 3D Text standard

- Non restricted fonts list

Create in CATIA V5 a new CATDrawing using your usual company standard and save it in a specific directory. This drawing must remain empty.

Warning: some fonts may leads to self intersecting string configuration that are not supported by the solid modeler. A typical example: the string “AAA” with space 0%.

  - Environment variables

Before using the 3D Text tool you need to set two environment variables: 3DTEXT_Temp

and 3DTEXT_Startup.

3DTEXT_Temp variable allows you to specify a directory where the temporary DXF file will be stored during the 3D Text creation process.  On Windows operating systems, proceed as explained below to set this variable:

- Select Properties command on My Computer icon.

- In the Advanced tab, click Environment Variables button and click New button (Figure 2.3 and 2.4)

- In the value field, specify a Directory where the DXF files will be stored (Figure 2.5)

Figure 2.3: accessing the environment variables panel

Figure 2.4: creating a new environment variable

Figure 2.5: valuating the 3DTEXT_Temp variable

3DTEXT_Startup variable allows you to specify the location of the start up drawing file (see chapter 2.1). Proceed as explained before to create this environment variable. In the Variable value field, enter the absolute path of your start up CATDrawing file (Figure 2.6).

Figure 2.6: valuating the 3DText_Startup variable

  - CATIA Settings

The unit of the DXF files to be imported must be set to millimeters. In CATIA V5, use Tools/Options command and in the General/Compatibility chapter select the DXF tab. In the Unit of the file field, select Millimeter (mm) (Figure 2.7). Let the other DXF settings with default values.

Figure 2.7: modifying the DXF unit settings

  - Adding more fonts

By default, two text fonts are available with the 3D Text tool: Arial (TrueType) and Monospac821BT (TrueType)

You can add more fonts using the following steps:

- In CATIA V5 open  Text_Solid_Powercopy_R18_v3.CATPart

- Edit the Font parameter (located under TextBody node)

- Select Multiple Values/Update Values command (Figure 2.8)

- Key-in the new exact font name(s)

- Save the CATPart document

These new fonts must be allowed by the drafting standard file associated with the start up drawing created in chapter 2.1. To check this point, open the start up drawing file and create a new Text.  Check the list of the allowed fonts in the Text Properties toolbar (Figure 2.9). If a font is missing, start CATIA in administrator mode and edit the standard file (Tools/Standard, drafting category, 3DText.xml standard, General/Allowed Text Fonts).

To get good quality results it is strongly advised to use only fonts such as TrueType fonts or PostScript fonts otherwise the 3D letter edges may not appear very smooth.

Figure 2.8: adding values to the Font parameter

Figure 2.9: checking the list of allowed fonts in the start up drawing

  - Using an icon to instantiate a Power Copy

- A Power Copy can be reused thanks to the Insert/Instantiate From Document command, using the catalog browser or using an icon pointing to a catalog item:

- Open CATIA V5 and create a new Catalog file (File/New command)

- Open Text_Solid_Powercopy_R18_v3.CATPart

- In the Catalog Editor workbench click Add tem icon

- In the Description Definition panel click Select external feature button and select the PowerCopy feature in the Text_Solid_Powercopy_R18_v3.CATPart window (Figure 2.10)

- On the catalog item use Publish Catalog Object command (Figure 2.11) and specify and alias name. This name will be displayed when dragging your mouse cursor on the icon.

- Save the Catalog file

- Open the Part Design Workbench

- Select the Tools/Customize command

- Switch to the Commands tab

- Select Catalogs

- Select the catalog item you wish to use and if you wish to change the icon, click Show Properties button and use the Icon button to select another icon from the list (Figure 2.12).

- Drag and drop the catalog item in an existing Part Design toolbar or in a new toolbar.

Figure 2.10: adding a Powercopy feature in a catalog

Figure 2.11: publishing a catalog item

- Figure 2.12: assigning an icon to a catalog item

- Using the 3D Text tool

  - 3D Text Creation

- To engrave or emboss 3D Texts in a part you will reuse a Power Copy feature. This Power Copy embeds some geometric features as well as some Knowledgeware features such Parameters, Rule, Reaction and Macro With Arguments.

- If you are working in assembly context, you must use the Open in new Window command on the desired part before using the 3D Text Power Copy.

- Use the Define In Work object command on the Body in which you want to create the 3D Text. This Body must be a “classical” body and not a Hybrid Design body.

- Instantiate the 3D Text Power Copy using the Insert/Instantiate From Document command and select “Text_Solid_Powercopy_R18_v3.CATPart”. You can also use an icon (see chapter 2.5).

- Select the Power Copy geometric inputs respecting the red arrow orientation (Figure 3.7):

- The support face or surface (planar or not)

- The anchor point (Point feature)

- The orientation line (Line feature or Edge)

- Click the Parameters button and enter values for the following parameters (Figure 3.8):

- The text size (mm)

- The text font

- The text type (embossing or engraving)

- The extrusion height (for embossing  text only)

- The pocket depth (for engraving text only)

- The text anchor point *

- The text string

- * To get better results on non planar faces support, it is advised to select “Middle Center” as Anchor_Point parameter value. Indeed the extrusion direction is computed thanks to a line normal to the selected face and going through the anchor point. This way you’ll get an “average” normal direction.

- Figure 3.7: selecting the Power Copy inputs

- Figure 3.8: valuating the 3DText_Solid Power Copy parameters

- After creation you will notice in the specifications tree that a TextBody Body is assembled to the current Body (Figure 3.9).

- Figure 3.9: specifications tree result after 3D Text Power Copy instantiation

- In the geometry area you will get a Pad (Figure 3.10) or a Pocket (Figure 3.11) depending on the Text_Type parameter value (Engraving or Embossing).

- Figure 3.10: embossing 3D Text geometric result

- Figure 3.11: engraving 3D Text geometric result

  - 3D Text modifications

- You can dynamically change the following parameter values:

- Extrusion_Height

- Pocket_Depth

- Text_Type

- The modification of the other 3D Text parameters is not fully automatic:

- In the specifications tree, modify the Anchor Point, Font and Size parameters if necessary.

- Define as current object the TextBody Body (Define In Work Object command) *

- Modify the Text Anchor_Point, Text_Size or Font parameter value.

- * If you forget to define the TextBody Body as the current object, an error message will be displayed and the update process will be aborted (Figure 3.13).

- Figure 3.13: warning message if current object is not a TextBody body

END of this document